Smart Pedestrian-Aware Traffic Light Controller in Verilog

# Objective

The goal of this homework is to design, implement, simulate, and demonstrate a Verilog-based traffic light controller for a two-road intersection. Unlike the class example, this controller must include a pedestrian crossing request and a night-mode blinking function.

You must submit:

- Verilog source code

- Verilog testbench

- Simulation waveform screenshots

- Short explanation of system behavior

# System Description

The system controls traffic lights for two streets:

- Main Road: Road A

- Side Road: Road B

The controller also supports:

- vehicle sensors for Road A and Road B;

- a pedestrian request button;

- night mode operation;

- reset functionality.

The design must be implemented as a finite state machine (FSM) in Verilog.

# Input Signals

| Signal | Description |
| --- | --- |
| clk | Clock signal |
| reset | Resets the system to the initial state |
| Sa | Vehicle sensor for Road A |
| Sb | Vehicle sensor for Road B |
| ped_req | Pedestrian crossing request |
| night_mode | Enables night blinking mode |

# Output Signals

| Signal | Description |
| --- | --- |
| Ga | Green light for Road A |
| Ya | Yellow light for Road A |
| Ra | Red light for Road A |
| Gb | Green light for Road B |
| Yb | Yellow light for Road B |
| Rb | Red light for Road B |
| Pwalk | Pedestrian walk signal |
| Pstop | Pedestrian stop signal |

# Required System Behavior Normal Mode

After reset, the controller starts with:

Road A = Green

Road B = Red

Pedestrian = Stop

Road A must remain green for at least 60 seconds.

After 60 seconds:

- if a vehicle is detected on Road B, Road A changes to yellow;

- after the yellow interval, Road A becomes red and Road B becomes green.

Road B must remain green for at least 40 seconds.

After 40 seconds:

- if there is no vehicle on Road B, the system returns to Road A green;

- if Road A has a vehicle waiting, the system also returns to Road A green;

- if Road B still has vehicles and Road A has no vehicles, Road B may remain green for an additional 10 seconds.

Whenever a road changes from green to red, it must pass through a yellow state first.

# Pedestrian Request Behavior

If ped_req = 1, the system must not immediately interrupt traffic.

The pedestrian request should be stored internally and served only after the current green phase is safely completed.

Before pedestrian crossing starts:

- the currently green road changes to yellow;

- both roads become red;

- Pwalk = 1 for 20 seconds;

- after pedestrian crossing, Pwalk = 0, Pstop = 1;

- the controller returns to Road A green.

During pedestrian crossing:

Road A = Red

Road B = Red

Pedestrian = Walk

# Night Mode Behavior

If night_mode = 1, the system enters night mode.

In night mode:

- normal traffic operation is suspended;

- yellow lights for both roads blink;

- green and red road lights are off;

- pedestrian signal remains stop.

When night_mode returns to 0, the controller must restart from the initial normal state:

Road A = Green

Road B = Red

Pedestrian = Stop

# Timing Requirements

For simulation, you may use a shortened time unit.

Use the parameter:

parameter TICKS_PER_10S = 1;

Timing table:

| Function | Duration |
| --- | --- |
| Road A minimum green | 60 seconds |
| Road B minimum green | 40 seconds |
| Yellow interval | 10 seconds |
| Pedestrian walk | 20 seconds |
| Optional Road B extension | 10 seconds |
| Night blinking interval | 10 seconds |

# Required FSM States

Students may use their own state names, but the design must include at least the following logical states:

| State | Meaning |
| --- | --- |
| A_GREEN | Road A green, Road B red |
| A_YELLOW | Road A yellow, Road B red |
| B_GREEN | Road A red, Road B green |
| B_YELLOW | Road A red, Road B yellow |
| ALL_RED_BEFORE_PED | Both roads red before pedestrian crossing |
| PED_WALK | Pedestrian walk signal active |
| NIGHT_BLINK_ON | Yellow lights ON in night mode |
| NIGHT_BLINK_OFF | Yellow lights OFF in night mode |

Verilog Implementation Requirements

The Verilog code must include:

- FSM state register

- Next-state logic

- Output logic

- Counter/timer logic

- Stored pedestrian request flag

- Night mode behavior

- Reset behavior

The code should be clean, readable, and well-commented.

# Testbench Requirements

The testbench must verify the following scenarios:

Scenario 1 — Normal Road A to Road B Transition

- Reset the system.

- Keep Road A green.

- Activate Sb.

- Verify that Road A changes to yellow and then Road B becomes green.

Scenario 2 — Road B Extension

- Keep Sb = 1.

- Keep Sa = 0.

- Verify that Road B green is extended.

Scenario 3 — Pedestrian Request

- Activate ped_req.

- Verify that the request is stored.

- Verify that pedestrian crossing starts only after the current road phase safely finishes.

- Confirm that both roads are red during pedestrian crossing.

Scenario 4 — Night Mode

- Activate night_mode.

- Verify that normal traffic is suspended.

- Verify that yellow lights blink.

- Deactivate night_mode.

- Verify that the system restarts from Road A green.

Scenario 5 — Reset

- Apply reset at any point.

- Verify that the system returns to its initial state.