Computer Architecture and Organization Lab

Final Project – MIPS-Based Digital Thermostat System

Project Goal

Design, implement, simulate, and deploy on FPGA a MIPS-based Digital Thermostat System capable of monitoring temperature and controlling a cooling fan and alarm system.

The processor must execute a program stored in instruction memory and make decisions based on temperature values stored in memory-mapped registers.

System Description

The system continuously monitors the current temperature and compares it with a user-defined target temperature.

Depending on the comparison result, the processor shall:

- Turn ON the cooling fan when temperature exceeds the desired threshold.

- Turn OFF the cooling fan when temperature returns below the threshold.

- Activate an alarm when temperature exceeds a critical level.

- Display the current system status using LEDs.

Memory-Mapped Registers

| Address | Function |
| --- | --- |
| 0x00 | Current Temperature |
| 0x04 | Desired Temperature |
| 0x08 | Critical Temperature |
| 0x0C | Fan Control Register |
| 0x10 | Alarm Register |

Required MIPS Instructions

The processor must support:

Arithmetic

- add

- sub

- addi

Logical

- and

- or

Memory

- lw

- sw

Branch

- beq

- bne

Jump

- j

Functional Requirements

Case 1

Current Temperature = 22°C

Desired Temperature = 25°C

Expected Result:

Fan = OFF

Alarm = OFF

Case 2

Current Temperature = 30°C

Desired Temperature = 25°C

Expected Result:

Fan = ON

Alarm = OFF

Case 3

Current Temperature = 45°C

Critical Temperature = 40°C

Expected Result:

Fan = ON

Alarm = ON

Required Hardware Modules

Processor Components

- Program Counter

- Instruction Memory

- Control Unit

- ALU

- Register File

- Data Memory

- Sign Extension

- Multiplexers

Thermostat Components

- Temperature Register

- Target Temperature Register

- Critical Temperature Register

- Fan Controller

- Alarm Controller

- LED Status Display

FPGA Inputs

| Input | Description |
| --- | --- |
| SW[7:0] | Current Temperature |
| BTN0 | Reset |
| BTN1 | Load Temperature |

FPGA Outputs

| Output | Description |
| --- | --- |
| LED0 | Fan Status |
| LED1 | Alarm Status |
| LED[7:2] | Temperature Status |

Testbench Requirements

Verify:

Normal Operation

- Temperature below threshold

Cooling Activation

- Temperature above threshold

Alarm Activation

- Temperature above critical threshold

Memory Access

- Correct lw/sw operation

Branch Logic

- Correct decision making